16-Port 10/100Mbps Fast Ethernet Switch

TRENDnet’s TE100-S16 is auto-sensing switch that automatically negotiates between 10Base-T and 100Base-TX network speeds. All ports support half- and full-duplex that allows maximum network performance.  It is designed to eliminate unnecessary traffic and relieve congestion on critical network paths.  Designed and built to increase network bandwidth, TE100-S16 is the best solution for both corporate and small business network environments.

TE100-S16

Features

- Compliant with IEEE 802.3 10Base-T, IEEE 802.3µ 100Base-TX

- 16 x 10/100Mbps Auto-Negotiation and Auto-MDIX Fast Ethernet Ports

- Supports Store and Forward Switching architecture with non-blocking full wire-speed performance

- Full/Half Duplex Transfer Mode for Each Port

- Wire-Speed Filtering/Forwarding Rates

- IEEE 802.3x Flow Control for Full-Duplex Mode

- Backpressure Flow Control for Half-Duplex Mode

- Integrate Address Look-Up Engine, Supports up to 8K Absolute MAC Address

- 512KB per Device RAM Buffer.

- Extensive Front-Panel Diagnostic LEDs

- Plug and Play Design

- Standard 19” (1U) Rack Mount Size (kit included)

- 5-Years Warranty

Specifications

| Hardware |  |
| --- | --- |
| Standard | IEEE 802.3 10Base-T IEEE 802.3u 100Base-TX ANSI/IEEE 802.3 Auto-Negotiation |
| Connectors | 16 x 10/100Mbps Auto-MDIX RJ45 Ports |
| Cabling | Cat. 3,4,5 UTP/STP for 10Base-T Cat. 5 UTP/STP for 100Base-TX |
| Supported Cable Length | 100 Meters max. |
| Protocols | CSMA/CD |
| Data Transfer Rate | 10Mbps: 10/20Mbps (Half/Full Duplex) 100Mbps: 100/200Mbps (Half/Full Duplex) |
| Data RAM Buffer | 512KB per device |
| Filtering Address Table | 8K Entries Per Device |
| Diagnostic LED | Power, Link/Activity, 100Mbps |
| Weight | 2.0 kg (4.4 lb) |
| Dimensions (W x H x D) | 440 x 140 x 44mm ( 17.3” x 5.51” x 1.73”) |
| Temperature | Operating: 0C ~ 40C (32F ~ 104F) Storage: -10C to 70C (14F to 158F) |
| Humidity | Operating: 10 % ~ 90 % (non-condensing) Storage: 5 % ~ 90 %(non-condensing) |
| Certifications | FCC, CE, VCCI |

TOP REASONS THE TE100-S16 IS A LASTING TREND

| Integration Friendly | Easily upgrade to Fast Ethernet without changing switch |
| --- | --- |
| Economical | Provides 10/100Mbps of Fast Ethernet in the same unit |
| Flexible | All Ports automatically negotiate 10/100 network speed, half/full modes, and Auto-MDIX connector type |
| Performance | Supports store and forward switching architecture with non-blocking full wire-speed performance |

Network Solution for TE100-S16

Package Contents	Related Product

- TE100-S16						- TE100-S16E+

- Multi-Language UG					- TE100-S24

- Power Cord						- TE100-S32+

- Rack Mount Kit

To order please call: 888-326-6061

3135 Kashiwa Street • Torrance, CA 90505 • (310) 891-1100 • Fax: (310) 891-1111

### Sales: sales@trendware.com • Support: techsupport@trendware.com

Tech Support Hot Line: (310)626-6252

### Official Site: www.TRENDNET.com