8-Port 10/100Mbps PoE Fast Ethernet Switch

TPE-S44 8-Port Smart Switch integrates Power over Ethernet (PoE) and network management features into a cost effective and scalable solution. The TPE-S44 reduces equipment cost by delivering data and power over existing network cable infrastructure.

TPE-S44 (A1.0R)

Features

- IEEE 802.3af Power over Ethernet (PoE) Standard Compliant

- 8 x 10/100Mbps Auto-Negotiation and Auto-MDIX Fast Ethernet RJ45 Ports

- Built-in 4 PoE Ports (Port 1 ~ Port 4)

- Supports PoE Power Maximum 15.4 watts for each POE port

- Supports PoE Powered Device(PD) classification identify

- Full Wire-Speed Non-Blocking Reception and Transmission

- Store and Forward Switching Method

- 96Kbytes RAM Data Buffer and 1K entries MAC Address Table

- Extensive Diagnostic LED’s on the Front Panel

- IEEE 802.3x Flow Control for Full-Duplex Mode; Back Pressure Flow Control for Half-Duplex Mode

- 5-Year Warranty

Specifications

| Hardware |  |
| --- | --- |
| Standard | IEEE 802.3 10Base-T IEEE802.3u 100Base-TX IEEE 802.3af Power over Ethernet |
| Network Media | 10Mbps: Cat 3,4,5 Unshielded Cable 100Mbps: Cat 5, 5E, 6 Unshielded Cable |
| Protocol / Topology | CSMA/CD ; Star |
| Ports | 4 x 10/100Mbps Auto-MDIX POE Ports (1~4) 4 x 10/100Mbps Auto-MDIX Fast Ethernet Ports (5 ~8) |
| Data Transfer Rate | Ethernet: 10/20Mbps (half/full duplex) Fast Ethernet: 100/200Mbps (half/full duplex) |
| RAM Buffer / Filtering | 96 Kbytes / 1K entry per device |
| Power | 48V DC 0.8A External Power Adapter |
| Diagnostic LEDs | Per Unit: Power, Power MAX Per Port: Link/Activity, Speed PoE: PoE Status, Link/Activity, Speed |
| Power Consumption | 6.3 watts max (no PD Device connected) 36.3 watts max (with 30w PoE Device connected) |
| Temperature | Operating: 0° ~ 40° C (32° ~ 104°F) Storage: -10° ~ 70° C (14° ~ 158°F) |
| Humidity | 10%- 90% (non-condensing) |
| Dimensions | ## 171 x 98 x 29mm (6.7 x 3.7 x 1.1in.) |
| Weight | ## 430g (13.8oz.) |
| Certification | FCC, CE |
| PoE |  |
| Power over Ethernet | Up to 15.4 watts per Port; PD classification Identify PoE Over Current Protection; PoE Circuit Sorting Protection PoE Power on RJ-45 pin 3,6 for Power+ and pin 1,2 for Power- |

Package Contents	Related Product

- TPE-S44						- TPE-101I

- Multi-Language User’s Guide	- TPE-102S

- Power Adapter (48V DC, 0.8A)		- TPE-S88

- TPE-S224WS

To order please call: 888-326-6061

20675 Manhanttan Place • Torrance, CA 90501 • (310) 961-5500 • Fax: (310) 961-5511

### Sales: sales@trendnet.com • Support: techsupport@trendnet.com

Tech Support Hot Line: 1-310-626-6252

### Official Site: www.TRENDNET.com