2-port USB 2.0 CardBus Adapter

The TU2-PCBUS offers two High Speed USB 2.0 ports designed for adding the new USB technology to existing Laptop/Notebook Computers. Computer users can instantly enjoy the latest USB 2.0 devices with 480Mbps transfer speed. The TU2-PCBUS is also backward compatible with existing USB 1.1 devices for extended compatibility. The TU2-PCBUS features CardBus PC Card Type II interface for maximum data transfer throughput between the host PC and the USB devices. Installation is a snap with TU2-PCBUS Plug-n-Play functionality. Additional USB 2.0 ports can be obtained by using the TU2-400, 4-port High Speed USB 2.0 hub, the ideal companion for the TU2-PCBUS.

TU2-PCBUS

# FEATURES

- 32-bit CardBus PC Card type II interface

- USB 2.0 specifications compliant and backwards compatible with USB 1.1

- Two USB 2.0 Ports

- Allows Simultaneous Operation of USB 1.1 and USB 2.0 Devices

- Supports high-speed (480Mbps), full-speed (12Mbps) and low-speed (1.5Mbps) data transfer modes

- Compatible with Windows 98SE/ME/2000/XP

- Plug-n-Play Installation

- Supports USB Hubs, Removable Drives, Digital Cameras, Scanners, CD-RW/DVD ROMs,

PC Video Cameras, Printers, CF/SM/MMC/SD/Memory Stick Readers/Writers and more.

- 3-year warranty

# TREND SPEC’S

Standard:		USB Specification 1.1, and 2.0

Data Speed:		High-Speed: 480 Mbps

Full-speed: 12 Mbps

Low-speed: 1.5 Mbps

Host Interface:		2-Port Hi-Speed USB 2.0 with Type-A Connectors.

Support Suspend Power Saving mode

Driver Support:		Windows 98SE/ME/2000/XP

Power Consumption:	3.3V DC, 1A max. (165mA @ idle)

Dimensions (WxHxD):	112 x 54 x 11 mm (4.4 x 2.13x 0.43 inches)

Weight:			40 g (1.4 oz.)

Operating Temp.: 	5˚ ~ 50˚ C (41˚ ~ 122˚ F)

Storage Temp.: 		-10˚ ~ 60˚ C (14˚ ~ 140˚F)

Operating Humidity:	20 % ~ 80 % non-condensing

Certifications:		FCC, CE

### TOP REASONS THE TU2-PCBUS IS A LASTING TREND

Compatibility:	    TU2-PCBUS is compatible with Windows 98SE/ME/2000/XP.

Flexibility:	    Supports both USB 2.0 Hi-Speed and USB 1.1 devices.

Installation:	    Easy installation with Plug-n-Play setup.

Speed:	 Supports 480Mbps High-Speed, 12Mbps Full-Speed, or 1.5Mbps Low-Speed USB operation mode.

TU2-PCBUS Application

TU2-PCBUS		2-port USB 2.0 CardBus Adapter

Product Information

TU2-400		4-port Hi-Speed USB Hub

To order please call: 888-326-6061

3135 Kashiwa Street • Torrance, CA 90505 • (310) 891-1100 • Fax: (310) 891-1111

Sales: sales@trendware.com • Support: techsupport@trendware.com	ss06042003