10/100Mbps TX to 100Base-FX

Multi-Mode Fiber Converter

TRENDnet’s TFC-110MSCE or TFC-110MSTE Fiber Media Converter transforms 10/100Base-TX UTP/STP media to 100Base-FX media and vice versa. The 10/100Base-TX Port Auto-Negotiating 10 or 100Mbps connection speed with Auto-MDIX media type. The fiber connection can be multi-mode SC or ST type.  This converter will give your Switch/Hub the ability to interface with fiber connection over a distance up to 2 km.

TFC-110MSCE

TFC-110MSTE

#### Features

- Compliant with IEEE 802.3 10Base-T and IEEE 802.3u 100Base-TX, 100Base-FX Standards

- Provides One Port 10/100Base-TX Auto Negotiation with RJ-45 Connector

- Provides One Port 100Base-FX Multi-Mode Fiber Optic with SC (TFC-110MSCE) or ST (TFC-110MSTE) Connector

- Auto MDI-X for 10/100Mbps-TX Port

- Auto-Negotiation for Speed and Duplex Mode on 10/100Mbps-TX Port

- Provides a slide Switch for Full and Half Duplex Selections on FX Port

- Hot Pluggable & Wall-Mountable

- 5-Year Warranty

#### Specifications (MMF: Multi-Mode Fiber)

| Model Name | Wavelength | Media | Pout | Sensitivity | Power |
| --- | --- | --- | --- | --- | --- |
| TFC-110MSCE | 1310nm | MMF | -19dBm | -32dB | -21dB |
| TFC-110MSTE | 1310nm | MMF | -19dBm | -32dB | -21dB |

| Hardware |  |
| --- | --- |
| Standard | IEEE 802.3 10Base-T IEEE 802.3u 100Base-TX, 100Base-FX |
| Network Media | ## 10Base-T: UTP Cat. 3, 4, 5, EIA/TIA-568 100-ohm STP 100Base-TX: UTP Cat. 5, EIA/TIA-568 100-ohm STP 100Base-FX: 50/125 um or 62.5/125 um Multi-Mode Fiber Optic Cable, Up to 2km |
| Protocol | CSMA/CD |
| Ports | 1 x 10/100Base-TX 1 x 100Base-FX |
| Transfer Rate | 10Base-T : 10Mbps/20Mbps (Half/Full Duplex) 100Base-TX : 100Mbps/200Mbps (Half/Full Duplex) |
| Diagnostic LED | Per Unit : Power Per Port : Link/Activity, Full-Duplex/Collision |
| Power Adapter | 7.5V DC, 1A External Power Adapter |
| Power Consumption | 2 watts (max) |
| Dimensions | 116 x 70 x 25mm (4.6 x 2.75 x 0.98 inches) |
| Weight | 200g (7 oz) |
| Temperature | Operating : 0 ~ 40 C (32 ~ 104 F) Storage : -25 ~ 70 C (-13 ~ 158 F) |
| Humidity | 5% ~ 90% RH |
| Certification | FCC,CE |

To order please call: 888-326-6061

3135 Kashiwa Street • Torrance, CA 90505 • (310) 891-1100 • Fax: (310) 891-1111

## Sales: sales@trendware.com  • Support: techsupport@trendware.com

Official Site: www.TRENDnet.com